# Debayan Roy, 11000125039

Assignment 2

## Objectives

- Half Subtractor using basic logic gates.

- Half Subtractor using NAND gates.

- Full Subtractor using basic logic gates.

- Full Subtractor using NAND gates.

- Full Adder using two Half Adders.

- Full Subtractor using two Half Subtractors.

A half subtractor subtracts one bit B from another bit A, giving DIFFERENCE (DIFF) and BORROW outputs. DIFF is 1 when A and B differ (XOR), and BORROW is 1 when A is 0 and B is 1 (A' · B).

| A | B | DIFF | BORROW |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |

Fig. A1 — Half Subtractor: XOR gate gives DIFF, NOT(A) AND B gives BORROW. Inputs A=0,B=0  DIFF=0, BORROW=0.

Since NAND is a universal gate, DIFF (=AB) and BORROW (=A·B) are both re-derived using NAND gates only, following the same 5-NAND XOR structure used for DIFF, plus a small NAND network for BORROW.

Fig. B1 — Half Subtractor built entirely with NAND gates.

A full subtractor also accepts a Borrow-IN (Bin) from a previous stage, producing DIFF and Borrow-OUT (Bout).

| A | B | Bin | DIFF | Bout |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |

Fig. C1 — Full Subtractor with basic gates

The full subtractor is re-derived using NAND gates only. As annotated in the simulation, key intermediate NAND terms include (AB) and C·(A ~ B)-type combinations, which are recombined using the NAND identity (XY) = X+Y (equivalent to OR of X, Y) to build Bout, while a cascaded NAND-XOR structure produces DIFF.

Fig. D1 — Full Subtractor implemented with NAND gates only

The first half adder adds A and B, giving an intermediate sum (AB) and carry (A·B). The second half adder adds this intermediate sum to Cin, giving the final SUM and a second carry. The two carries are OR-ed to give Cout.

Fig. E1 — Full Adder from two cascaded Half Adders

The first half subtractor computes AB, giving intermediate difference (AB) and borrow (A·B). The second half subtractor subtracts Bin from this intermediate difference, giving the final DIFF and a second borrow. The two borrow terms are OR-ed to give Bout.

Fig. F1 — Full Subtractor from two cascaded Half Subtractors

All subtractor circuits (half and full) were successfully designed and simulated using both basic gates and NAND-only realizations, and the composite Full Adder / Full Subtractor circuits built from two half-blocks. Simulated outputs matched the theoretical truth tables in every case, confirming correct design and the universality of the NAND gate.