ASSIGNMENT 6

Design and Implementation of a

4-Bit Carry Lookahead Adder

# NAME: DEBAYAN ROY; BTECH CSE (SEM III); ROLL: 11000125039

In a ripple carry adder, each full adder must wait for the carry from the previous stage, so the total delay grows with the number of bits. A carry lookahead adder (CLA) removes this waiting. Instead of passing the carry from stage to stage, it calculates every carry directly from the input bits Ai, Bi and the initial carry C0, using extra "look-ahead" logic. All carries are therefore produced at nearly the same time, and the addition is much faster.

The method is based on two signals defined for every bit position i:

- Generate, Gi = Ai·Bi: the stage produces a carry by itself, whatever the incoming carry is.

- Propagate, Pi = Ai ⊕ Bi: the stage passes an incoming carry on to the next stage.

## Carry equations

For a full adder, the carry-out of stage i can be written in terms of Gi and Pi:

Ci+1 = Gi + Pi·Ci

Substituting the previous carry repeatedly removes the dependence on the intermediate carries, so every carry depends only on the G, P and C0 values:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

Each of these is a sum of AND terms, so it can be built with one level of AND gates followed by an OR gate. Because no carry depends on another carry, all four are calculated in parallel.

## Sum equation

Once the carries are known, each sum bit needs only one XOR gate:

Si = Ai ⊕ Bi ⊕ Ci = Pi ⊕ Ci

The final carry-out of the adder is C4.

## Block structure

The adder is built from three blocks:

- Propagate/generate block: four XOR gates give P3–P0 and four AND gates give G3–G0.

- Carry lookahead block: AND and OR gates implement the expressions for C1 to C4 directly from Pi, G and C0.

- Sum block: four XOR gates combine each Pi with its carry Ci to give S3–S0.

# 5. Circuit Implementation

The circuit was built in a logic simulator as shown in Figure 1. The inputs A3–A0 and B3–B0 are at the top, with each bit pair feeding one AND gate (Gi) and one XOR gate (Pi). The carry lookahead logic sits in the middle of the circuit, and the carries C1–C4 are tagged in the diagram. The sum XOR gates and the output displays are at the bottom.

Figure 1: 4-bit carry lookahead adder (C0 is the carry-in; the output marked CARRY is C4)